Dashboard Chrome Extension

Earth Funeral Las Vegas  —  Facility Operations

Feature & Architecture Reference

Version 3.1.0  |  Last Updated: May 2026

About This Document

This reference document describes every feature of the Dashboard Chrome Extension, how data flows between components, and how external services are integrated. It is intended to be updated whenever changes are made to the extension. Upload or open in Google Docs to edit collaboratively.

# 1. Project Overview

The Dashboard Extension is a Chrome Extension (Manifest V3) that provides real-time monitoring and automation for the vessel racks at Earth Funeral Las Vegas. It captures live telemetry from the HMI (Human-Machine Interface) system, displays a visual grid of all 39 vessel slots, and runs automated recovery logic to keep vessels running with minimal operator intervention.

Key capabilities at a glance:

- Real-time telemetry display (temperatures, airflow, pressure, motor, mass)

- Setpoint injection directly into the HMI WebSocket (no UI clicking required)

- Watchdog automation engine — escalating fault recovery runs every 2 minutes

- Cross-machine synchronization via Google Drive

- Google Sheets logging of all setpoint changes and motor restarts

- Kiosk mode with animated weather, trivia, news, and Slack tickets

- Fleet sparkline chart showing issue trends in real time

# 2. File Structure

All source files live in the extension root directory. There is no build step — files are loaded directly by Chrome.

| File | Size | Purpose |
| --- | --- | --- |
| background.js | 137 KB / 2,959 lines | Service worker: telemetry processor, watchdog engine, Drive/Sheets sync, HMI command injection |
| dashboard.js | 177 KB / 4,509 lines | Main UI: vessel grid rendering, setpoint controls, issue filters, kiosk mode, real-time updates |
| dashboard.html | 8 KB | HTML shell: grid containers, buttons, panels |
| dashboard.css | 58 KB | Complete styling: grid layout, cards, dark/light theme, kiosk mode animations |
| content.js | 12 KB | Content script: HMI DOM scraper, reads temp-control switch state |
| ws-bridge.js | 1.8 KB | Isolated-world relay: bridges MAIN-world WebSocket events to background.js |
| drive-sync.js | 24 KB | Google Drive/Sheets API module: JWT auth, file sync, setpoint logging |
| options.html | 2.3 KB | First-time setup page for credentials |
| options.js | 1.3 KB | Credential storage handler (Google service account, Slack token) |
| tweet-sizer.js | 455 B | Utility: auto-scales embedded Twitter iframes |
| widgets.js | 8 B | Empty stub (deprecated) |
| manifest.json | 1.6 KB | Chrome Extension MV3 configuration |

# 3. Manifest & Permissions

manifest.json declares Manifest Version 3 (MV3). The service worker is background.js loaded as an ES module.

## 3.1 Chrome Permissions

| Permission | Purpose |
| --- | --- |
| tabs | Required to find the active HMI tab and inject commands |
| scripting | Allows background.js to inject scripts into HMI tabs for automation |
| storage | Stores credentials, setpoints, and watchdog state locally |
| alarms | Triggers periodic watchdog tick every 2 minutes |
| declarativeNetRequest | Modifies network request headers for cross-origin API calls |

## 3.2 Host Permissions

The extension is granted access to the following external hosts:

| Host | Purpose |
| --- | --- |
| atlas.earthfuneral.com | HMI REST API — vessel configs, temp setpoints, motor speed |
| fh.earthfuneral.com | Facility host API |
| grafana.earthfuneral.tech | Uptime metrics queries (SQLite datasource) |
| oauth2.googleapis.com | Google OAuth token endpoint |
| sheets.googleapis.com + www.googleapis.com | Google Sheets API (setpoint logs, motor audit log) |
| api.open-meteo.com | Weather data for kiosk mode (public, no auth) |
| news.google.com | News feed for kiosk mode |
| opentdb.com | Trivia questions for kiosk mode |
| v2.jokeapi.dev | Dad jokes for kiosk mode |
| slack.com | Fetch open support tickets from #general-tickets |
| x.com / twitter.com / xcancel.com / nitter.* | Social feed panels for kiosk mode |

## 3.3 Content Scripts

Two content scripts are injected on HMI pages (https://atlas.earthfuneral.com/internal/hmi/*):

- ws-bridge.js — runs in the ISOLATED world, relays WebSocket messages from the MAIN world to background.js

- content.js — scrapes temp-control switch state from the DOM and listens for hmi:scrape requests

- Both run at document_start and are injected in all_frames

# 4. Architecture & Data Flow

## 4.1 Component Overview

There are four distinct execution contexts that communicate via Chrome message passing:

- HMI Page (MAIN world) — the live HMI application with its WebSocket/EventSource telemetry stream

- HMI Page (ISOLATED world) — ws-bridge.js hooks the MAIN-world WebSocket and relays events; content.js scrapes the DOM

- Service Worker (background.js) — the central hub: receives telemetry, runs the watchdog, maintains state, orchestrates Drive/Sheets sync

- Dashboard Tab (dashboard.js + dashboard.html) — the operator-facing UI, fetches state from the service worker, sends commands

## 4.2 WebSocket Hooking

The extension captures HMI telemetry by wrapping window.WebSocket in the MAIN world. This intercepts all send() and onmessage events without modifying the HMI application. The ISOLATED world (ws-bridge.js) relays these events to background.js using chrome.runtime.sendMessage. This same channel is used in reverse to inject setpoint commands silently — background.js sends a command payload, ws-bridge.js calls the original WebSocket send(), and the HMI receives the command as if a user typed it.

## 4.3 Hidden Popup Automation

Some HMI actions (motor reset buttons, rack pause toggles) cannot be triggered via WebSocket — they require clicking DOM elements. The watchdog engine handles these by opening a hidden popup window pointing to the HMI vessel detail page, injecting a script to click the target button, then closing the popup immediately. A queue (_motorCardResetQueue) serializes these operations since the HMI shows only one card at a time.

## 4.4 Service Worker Keep-Alive

MV3 service workers suspend after approximately 30 seconds of inactivity. dashboard.js opens a chrome.runtime port (keepAlive) before any long operation and disconnects it after receiving the response. This prevents premature suspension during watchdog ticks or Drive sync operations.

# 5. Vessel Grid (dashboard.js)

## 5.1 Layout

The grid displays 13 columns (racks, numbered 001-013) with 3 rows per column (A, B, C vessel positions), yielding 39 slots maximum. Each column has a header with a pause button. Unracked vessels are listed in a separate panel.

## 5.2 Vessel Card Contents

Each card shows:

- Status bubble — Running (green), Off (black), Stopped (red), Fault (red cross)

- Slot ID and Vessel number

- Process temp (converted from °C to °F for display)

- Heater temp, probe temps (temp0, temp1, temp2), average temp

- Motor angle (current rotation in degrees)

- Mass (converted from kg to lbs)

- Airflow (l/min)

- Pressure (kPa)

- Mixer module status and valve module status

- Blower state and declog flags(Int Declog Event, Exh Declog Event)

- Issue badges (Motor, Temp, Airflow, Pressure, Probe Delta)

## 5.3 Telemetry Updates

The dashboard polls background.js for the latest dashboardState on a short interval. Updates are scheduled via requestAnimationFrame to avoid render conflicts. When a hidden popup is open for watchdog automation, rendering is paused to prevent interference.

# 6. Setpoint Control

Operators click any vessel card to open a setpoint editor popup. Changes are sent via the setpoint:set message to background.js, which validates the value, finds the active HMI tab, and injects the command into the HMI WebSocket.

## 6.1 Controllable Setpoints

- Temperature  —  32-170 °F

- Motor Speed  —  0-60 R/hr (rotations per hour)

- Blower Airflow  —  0-65 l/min

- Temp Control Toggle  —  ON / OFF

## 6.2 Logging

Every setpoint change (watchdog-generated) is appended to the Google Sheets Watchdog Setpoint Log with: timestamp, vessel ID, slot name, setpoint type, old value, and new value.

# 7. Watchdog Automation Engine

The watchdog runs _watchdogTick() every 2 minutes via chrome.alarms. It only executes on the designated owner machine (IP 192.168.50.176 currently the PC running the kiosk) to prevent multiple machines from issuing conflicting commands simultaneously.

## 7.1 Recovery Sequence

The watchdog evaluates every vessel slot in order and applies the first matching condition:

| # | Condition | Action |
| --- | --- | --- |
| 1 | Valve fault / mixer faulted | Opens hidden popup to click HMI reset, injects motor speed setpoint to restart |
| 2 | Motor unexpectedly stopped — attempt 1 | Send speed setpoint bumped by 1 from desired (e.g., send 59 if desired is 60) |
| 3 | Motor unexpectedly stopped — attempt 2 | Fault-reset via hidden popup click + re-send desired speed (e.g., 60) |
| 4 | Motor unexpectedly stopped — attempt 3+ | Full rack reset: close and reopen HMI panel |
| 5 | 3 failed restart attempts on a slot | Rack reset + 10-minute cooldown before further attempts |
| 6 | Temp control found OFF | Re-enables temp control at last-known setpoint (skips if setpoint has been reached) |
| 7 | Blower found OFF | Resends airflow setpoint (skips if declog active or pressure > 9 kPa) |

## 7.2 Skip Conditions (Watchdog Will Not Act)

- Rack is manually paused OR automatically paused (HMI auto-pauses if A position is empty)

- Vessel state is power_inactive, wait_vessel, or ctrl_inactive

- Probe delta alarm active (probe span > 8 °F — HMI blocks temp control anyway)

- Declog is active (intake or exhaust clearing in progress)

- Pressure > 9 kPa for airflow-related actions

- Span mitigation is active for temp control actions

## 7.3 Logging

All watchdog actions are appended to the Watchdog Setpoint Log sheet. Motor restart attempts are additionally recorded in the Motor Audit Log tab (includes slot name, attempt number, step description, outcome). Rows older than 72 hours are automatically purged from the temp/blower setpoint log but not the motor audit log.

# 8. Issue Detection & Filtering

## 8.1 Issue Types

| Issue Type | Detection Logic |
| --- | --- |
| Motor | mixer_faulted OR mixer_stopped OR valve_fault flag set |
| Temp | Process temp >= 10 °F above current temperature setpoint |
| Airflow | airflow < 5 l/min OR blower requested but not flowing |
| Pressure | pressure > 9 kPa (potential blockage risk) |
| Probe Delta | All 3 probes present + span mitigation active + max probe minus min probe >= 8 °F |

## 8.2 UI Modes

- Normal grid — all 39 slots visible

- Issues mode — only vessels with at least one active issue visible

- Filter chips — toggle individual issue types (Motor / Temp / Airflow / Pressure / Probe Delta)

- Vessel search — highlights cards matching a vessel number

# 9. Fleet Sparkline Chart

A real-time chart rendered in the bottom-right panel tracks the total number of active issues across the entire fleet. It maintains 180 data points (one every 10 seconds = 30 minutes of history). The chart shows the current count, a rolling average, and uses color coding to indicate severity. It is always visible in both normal and kiosk modes.

# 10. Rack Column Controls

- Pause Individual Rack — toggles the reset rack button in the HMI footer for that column; watchdog skips paused racks

- Pause All — sends pause command to all 13 columns simultaneously

- Restart All — clears all pauses

- Stop — halts any active restart automation immediately

Rack pause state is scraped from the HMI every 5 minutes or on-demand. The watchdog distinguishes between manually paused racks and racks auto-paused by the HMI due to an empty A position.

# 11. Kiosk Mode

Kiosk mode is intended for a dedicated facility display screen. It cycles through entertainment panels while always keeping the fleet sparkline and attention list visible at the bottom of the screen. Iterates every 15 seconds for each slide.

## 11.1 Rotating Panels

| Panel | Description |
| --- | --- |
| Weather | Real-time conditions via Open-Meteo API; animated background (rain streaks, lightning, fog, stars based on conditions) |
| Trivia | Random daily question from Open Trivia DB with multiple-choice answers |
| Jokes | Dad jokes from Joke API |
| News | Google News feed embedded |
| Slack Tickets | Live list of open support tickets from #general-tickets Slack channel |

## 11.2 Always-Visible Bottom Bar

- Fleet sparkline chart (issue trend over 30 minutes)

- Attention list — top 3 most critical issues with vessel name and issue type

- Quick-stat summary — uptime percentage, vessels running, total active issues

## 11.3 Controls

- Enter kiosk mode — click the TV button in the dashboard legend

- Exit kiosk mode — click the same button or use the keyboard shortcut

- Theme toggle — light / dark mode (persists to localStorage)

- Grid flip — reverses rack order so rack 13 appears on the left

- Zoom/fit — scales the grid to fit all vessels on screen

# 12. Cross-Machine Synchronization

Watchdog enabled/disabled state is shared across all operator machines via a JSON file in Google Drive. Any machine can toggle the watchdog; the change is written to Drive and all other machines pick it up within approximately one minute.

## 12.1 Drive File

- File name: watchdog_sync.json

- Google Drive folder ID: 1s5zwq7MIZ5AaqmNQzkx66jIbMcVuEvax

- Contents: { enabled, changedBy, changedAt, ownedBy, time }

## 12.2 Authentication

The extension uses a Google Cloud service account with the Drive and Sheets scopes. Credentials are stored in chrome.storage.local (machine-local only, never synced to a Google account). The extension signs its own JWT tokens and exchanges them for access tokens — no OAuth user consent flow is required.

- SA_EMAIL — service account email (chrome.storage.local)

- SA_KEY — private key in PEM format (chrome.storage.local)

- SLACK_TOKEN — Slack bot token (chrome.storage.local)

- Token expiry: 1 hour; auto-refreshed on next use

# 13. Google Sheets Logging

Two tabs in the shared Google Sheet are maintained by the extension:

- Watchdog Setpoint Log — every setpoint change (manual and automated): timestamp, vessel, slot, setpoint type, value, source

- Motor Audit Log — motor restart attempts: slot name, attempt number, step description, timestamp, outcome

Rows older than 72 hours are automatically purged from the setpoint sheet keep the sheet manageable.

# 14. Message Passing Reference

All inter-component communication uses chrome.runtime.sendMessage / chrome.runtime.onMessage. The following messages are defined:

| Message | Direction | Description |
| --- | --- | --- |
| WS_DATA | content.js / ws-bridge.js → background | Telemetry frame from HMI WebSocket (temps, pressures, airflow, etc.) |
| WS_CTRL | content.js / ws-bridge.js → background | Outgoing HMI command (logged for replay) |
| hmi:scrape | content.js → background | Temp-control switch state scraped from open panel DOM |
| hmi:get-vessel-map | background → content.js | Look up vessel name → slot mapping |
| dashboard:get | dashboard.js → background | Fetch full dashboardState + unracked vessels |
| dashboard:request-pause-sync | dashboard.js → background | On-demand rack pause state sync |
| dashboard:refresh-setpoints | dashboard.js → background | Trigger API polling for updated setpoints |
| setpoint:set | dashboard.js → background | Inject setpoint command into HMI WebSocket (temp / motor / blower / temp-ctrl) |
| pause-rack | dashboard.js → background | Toggle column pause in HMI footer (dialog automation) |
| watchdog:toggle | dashboard.js → background | Enable / disable watchdog automation (syncs to Drive) |
| reset-slot-card | dashboard.js → background | Click motor reset button in HMI vessel card via hidden popup |
| xcancel:fetch-page | dashboard.js → background | Fetch X/Twitter feed via xcancel.com |
| nitter:fetch-rss | dashboard.js → background | Fetch Nitter RSS (privacy-friendly Twitter) |
| slack:fetch-tickets | dashboard.js → background | Fetch open Slack tickets from #general-tickets |

# 15. External Services

| Service | Endpoint | Purpose | Auth |
| --- | --- | --- | --- |
| Atlas REST API | atlas.earthfuneral.com/api/ | Vessel configs, setpoints | None (same-origin) |
| Grafana | grafana.earthfuneral.tech | Uptime metrics (SQLite datasource) | Basic auth (stored in chrome.storage) |
| Google Drive v3 | googleapis.com/drive/v3/ | Read/write watchdog_sync.json | JWT (service account) |
| Google Sheets v4 | sheets.googleapis.com | Setpoint log, motor audit log | JWT (service account) |
| Open-Meteo | api.open-meteo.com | Weather data (kiosk) | None (public) |
| Slack | slack.com/api/conversations.history | Open ticket list | Bearer token |
| Google News | news.google.com | News feed (kiosk) | None |
| Trivia DB | opentdb.com | Random trivia questions | None |
| Joke API | v2.jokeapi.dev | Dad jokes | None |
| X/Twitter proxies | xcancel.com, nitter.*, twitter.com | Social feed (kiosk) | None |

# 16. State Data Structure

background.js maintains dashboardState — an object keyed by slot name (e.g., "001A") with one entry per vessel slot. Each entry contains:

- slotName — rack + position identifier (e.g., 001A, 013C)

- vesselId — numeric database ID

- vesselName — case/vessel name string

- vesselPresent — boolean: true if a vessel is racked

- mechanicalStatus — RUNNING, FAULT, wait_vessel, ctrl_inactive, etc.

- valveModuleStatus — ON / OFF

- mixerModuleStatus — MIXER_RUNNING, MIXER_STOPPED, MIXER_FAULTED

- tempModuleStatus — temp_control_normal, etc.

- intake_declog_req / exhaust_declog_req — 0 or 1 (declog active flags)

- rackGroupPaused — boolean: true if the column is paused in the HMI footer

- setpoints — { tempSp (°F), motorSp (R/hr), airflowSp (l/min) }

- telemetry — { processTemp, heaterTemp, temp0-2, currentAngle, mass, airflow, pressure, lastUpdate }

- motorUptimePct — percentage from Grafana

- lastDeclogClearedAt / lastAirflowPositiveAt — timestamps for guard logic

A separate unrackedVessels array displays vessels that are not currently assigned to any slot.

# 17. UI System (dashboard.css)

## 17.1 Theming

CSS variables control all colors (--bg, --text, --card-bg, --header-bg, --border). Light mode is the default. Dark mode is applied by toggling a class on the document root. The preference is saved to localStorage.

## 17.2 Layout

- Card size: 230 px wide, variable height

- 13 columns (racks) x 3 rows (A/B/C)

- Flex layout with auto-wrap

- Fixed header: title, logo, controls, attention list, filter bar

# 18. Setup & Configuration

## 18.1 First-Time Setup

On first install, navigate to the extension options page (options.html). Three fields are required:

- Google Service Account Email

- Google Service Account Private Key (PEM format)

- Slack Bot Token

These are saved to chrome.storage.local. They are machine-local and are not synced to any Google account.

## 18.2 Watchdog Ownership

The watchdog automation engine only executes on the machine with local IP 192.168.50.176 (currently hardcoded in background.js). All other machines read watchdog state from Drive but do not execute commands. To change the owner machine, update the IP constant in background.js.

## 18.3 Google Cloud Setup

The service account must have:

- Google Drive API enabled — read/write access to the shared folder

- Google Sheets API enabled — append/read access to the log spreadsheet

- The shared Drive folder ID and Sheet ID are hardcoded in drive-sync.js

# 19. Security Notes

- All secrets (service account key, Slack token) are stored in chrome.storage.local — machine-local only

- No inline scripts in HTML; no eval() usage; all JS loaded as external files

- Slack token is stored as plaintext (no encryption at rest in chrome.storage.local)

- The extension has broad host permissions; limit extension installation to facility-managed machines

# 20. Known Quirks & Implementation Notes

- HMI vessel IDs can be a string slot name (e.g., 001A) or a numeric database ID — both are handled

- Heater control is declared OFF if no heartbeat is received for 30 seconds

- Motor retry escalation: after 3 failed restarts, a full rack reset is triggered with a 10-minute cooldown

- The HMI auto-pauses a rack when the A position is empty; the watchdog distinguishes this from a manual pause

- Declog mitigation: watchdog skips airflow setpoint sends during active declog cycles with a 3 minute cooldown to allow airflow to ramp up

# Change Log

Use this section to record changes to the extension over time.

| Version | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 3.1.0 | May 2026 |  | Initial documentation of current codebase |